# GreenPulse Wiring and RGB Indicator Guide

Connect the OLED, environmental sensors and two HW-479 RGB modules to the ESP32 using the pin map below. RGB 1 reports plant conditions; RGB 2 reports Wi-Fi and MQTT connection status.

Use GPIO numbers printed on a classic ESP32 Dev Module, not physical pin positions. Disconnect USB power before changing wires. All components share ESP32 GND.

# 1 Sensor and display connections

| Component | Power pins | Signal pins and other connections |
| --- | --- | --- |
| DHT22 or AM2302 | VCC / + → 3V3 GND / − → GND | DATA / S / OUT → GPIO4 |
| SSD1306 OLED 128 × 64 I2C | VCC → 3V3 GND → GND | SDA → GPIO21 SCL → GPIO22 |
| BH1750 light sensor GY-302 module | VCC → 3V3 GND → GND | SDA → GPIO21; SCL → GPIO22 ADDR → GND if exposed, for address 0x23 |
| Resistive soil sensor Blue interface board | VCC → 3V3 GND → GND | AO / A0 → GPIO34 DO / D0 → leave unconnected |
| Soil probe | Via interface board | Connect the two probe wires to the board’s two-pin probe connector. Do not connect the probe directly to an ESP32 GPIO. |

OLED and BH1750 share GPIO21 and GPIO22. Join each shared signal on a breadboard row. The OLED address is normally 0x3C or 0x3D; the sketch checks both.

For a bare four-pin DHT22, with the grille facing you and pins down: 1 = VCC, 2 = DATA, 3 = unused, 4 = GND. Add a 10 kΩ resistor between DATA and 3V3 if the sensor board has no pull-up resistor.

# 2 Connections for both RGB modules

| Module pin | RGB 1 plant status | RGB 2 connection status |
| --- | --- | --- |
| R | GPIO25 | GPIO18 |
| G | GPIO26 | GPIO19 |
| B | GPIO27 | GPIO23 |
| − / common cathode | GND | GND |

This wiring is for common-cathode modules marked R, G, B and −, driven HIGH to light a channel. Check the labels rather than assuming a header order. A common-anode (+) version needs different wiring and inverted logic. Each colour needs a current-limiting resistor; if these are absent on the board, add 330 Ω in series with each R, G and B wire.

# 3 RGB 1 plant condition colours

Label the first module PLANT. The most severe air or soil condition wins: critical overrides warning, and warning overrides normal. Yellow uses red and green together.

| Colour | Meaning | User action |
| --- | --- | --- |
| Green · steady | Normal within the configured air and soil limits | Continue monitoring. |
| Yellow · steady | Warning, or soil calibration is required | Read the bottom OLED message and check the relevant condition. |
| Red · steady | Critical air or dry-soil condition, or a DHT22 / BH1750 error | Read the OLED message. Check the plant or sensor connection before acting. |
| Blue | Not used for plant status | Reserved for other uses; no current plant meaning. |
| Off | Possible briefly during startup | If it stays off, check power, wiring and the sketch. |

# 4 Current plant thresholds

| Reading | Green normal | Yellow warning | Red critical |
| --- | --- | --- | --- |
| Soil moisture | 40% to 100% | 20% to below 40% | Below 20% |
| Air temperature | 18°C to 30°C | 10°C to below 18°C; above 30°C to 35°C | Below 10°C or above 35°C |
| Air humidity | 40% to 70% RH | 25% to below 40%; above 70% to 85% | Below 25% or above 85% |

These are demonstration thresholds, not universal plant care targets. DHT22 measures air temperature and humidity; it does not measure soil temperature. BH1750 lux is displayed, but no low-light or high-light thresholds currently affect the plant LED.

# 5 Soil percentage and OLED messages

The example calibration uses dry raw = 4095 and watered-and-drained raw = 1500. Replace them with measurements from your own probe and soil. Until then, the displayed “% demo” is an estimate. This is a relative moisture scale, not a laboratory water-content measurement.

| OLED message | What it means |
| --- | --- |
| NORMAL | All evaluated plant conditions are normal. |
| SOIL DRY / SOIL VERY DRY | Soil warning / soil critical. |
| AIR WARNING / AIR CRITICAL | Air temperature or humidity is outside the normal range. |
| AIR + SOIL WARNING / CRITICAL | Both air and soil have the named severity. |
| SOIL: CALIBRATE | Calibration is missing or invalid; normally yellow unless air is critical. |
| DHT22 ERROR / BH1750 ERROR / SENSOR ERROR | One or both environmental sensor readings have failed; red. |

Too wet: the current code does not raise an overwatering warning. A 100% reading can still show green. A raw value stuck at 4095 can also indicate a connection or signal problem; verify that the reading changes between dry and wet soil.

# 6 RGB 2 connection status colours

Label the second module CONNECTION. Its colours describe communications and are independent of the plant LED. For example, PLANT red with CONNECTION blue means a plant alert while Wi-Fi is connected in Wi-Fi-only mode.

| Colour and pattern | Meaning |
| --- | --- |
| Blue · blinking | Initial Wi-Fi connection attempt, during the first 30 seconds before the first successful connection. Approximately 0.5 seconds on and 0.5 seconds off. |
| Blue · steady | Wi-Fi connected with MQTT disabled or incompletely configured. Also appears while waiting to begin the first MQTT attempt, including time synchronisation. |
| Green · steady | Wi-Fi and the MQTT broker are connected. |
| Yellow · steady | Wi-Fi connected, but MQTT is attempting to connect, retrying, failed or disconnected after an attempt. |
| Red · steady | Wi-Fi is disconnected after the initial 30-second window, or the connection has been lost. It can remain red while reconnecting. |
| Off | Possible briefly at boot. If persistent, check power, wiring and software. |

With ENABLE_MQTT set to false, steady blue is the expected connected state. Green requires valid MQTT configuration and a successful broker connection. Green confirms the connection only; it does not prove sensor data was published or saved by a backend.

# 7 Connection labels on the OLED

| OLED label | Meaning |
| --- | --- |
| W:OFF | Wi-Fi disconnected. |
| W:ON | Wi-Fi connected; MQTT not configured or disabled. |
| M:ON | MQTT connected. |
| M:OFF | MQTT configured but not connected. |

# 8 Final wiring and startup checks

Power the ESP32 through its USB connector. Use its 3V3 rail for the listed sensor modules, and connect every GND to a common ground. Do not feed a 5 V analogue signal into GPIO34.

Keep the soil interface board and electrical connectors dry. Only the probe prongs go into soil. Follow the printed board labels for the connector order.

Confirm that GPIO18, GPIO19 and GPIO23 are used only by RGB 2. Any future pump, float switch or soil-temperature sensor must use separate suitable pins.

Check breadboard rail continuity and tighten loose jumper wires. Reconnect USB and allow the sensors to start; readings normally update every 2.5 seconds.

Check that soil raw readings change before trusting the percentage. Calibrate using dry soil and watered soil after excess water has drained.

Check the PLANT colour against the OLED condition message, then check the CONNECTION colour against its Wi-Fi or MQTT label.

Scope: this guide covers the display, DHT22, BH1750, resistive soil-moisture sensor and two RGB modules. Pump control, a water-level switch and a dedicated soil-temperature probe are not included in these connections.