# GreenPulse Wiring and RGB Indicator Guide

Connect the OLED, environmental sensors, dedicated soil-temperature probe and two HW-479 RGB modules to the ESP32 using the pin map below. RGB 1 reports plant conditions and sensor faults; RGB 2 reports Wi-Fi and MQTT connection status.

Use GPIO numbers printed on a classic ESP32 Dev Module, not physical pin positions. Disconnect USB power before changing wires. All components share ESP32 GND.

# 1 Sensor and display connections

| Component | Power pins | Signal pins and other connections |
| --- | --- | --- |
| DHT22 or AM2302 | VCC / + → 3V3 GND / − → GND | DATA / S / OUT → GPIO4 |
| SSD1306 OLED 128 × 64 I2C | VCC → 3V3 GND → GND | SDA → GPIO21 SCL → GPIO22 |
| BH1750 light sensor GY-302 module | VCC → 3V3 GND → GND | SDA → GPIO21; SCL → GPIO22 ADDR → GND if exposed, for address 0x23 |
| Resistive soil sensor Blue interface board | VCC → 3V3 GND → GND | AO / A0 → GPIO34 DO / D0 → leave unconnected |
| Soil probe | Via interface board | Connect the two probe wires to the board’s two-pin probe connector. Do not connect the probe directly to an ESP32 GPIO. |
| DS18B20 soil-temperature sensor 3-wire, externally powered | VDD / VCC → 3V3 GND → GND | DQ / DATA → GPIO32 4.7 kΩ pull-up from DQ to 3V3 |

OLED and BH1750 share GPIO21 and GPIO22. Join each shared signal on a breadboard row. The OLED address is normally 0x3C or 0x3D; the sketch checks both.

For a bare four-pin DHT22, with the grille facing you and pins down: 1 = VCC, 2 = DATA, 3 = unused, 4 = GND. Add a 10 kΩ resistor between DATA and 3V3 if the sensor board has no pull-up resistor.

The DS18B20 is used as a dedicated soil-temperature probe on GPIO32. Use three-wire powered mode and a 4.7 kΩ pull-up resistor between DQ and 3V3. The sketch rejects missing, stale and invalid probe readings (including the DS18B20 power-up value of 85°C).

# 2 Connections for both RGB modules

| Module pin | RGB 1 plant status | RGB 2 connection status |
| --- | --- | --- |
| R | GPIO25 | GPIO18 |
| G | GPIO26 | GPIO19 |
| B | GPIO27 | GPIO23 |
| − / common cathode | GND | GND |

This wiring is for common-cathode modules marked R, G, B and −, driven HIGH to light a channel. Check the labels rather than assuming a header order. A common-anode (+) version needs different wiring and inverted logic. Each colour needs a current-limiting resistor; if these are absent on the board, add 330 Ω in series with each R, G and B wire.

# 3 RGB 1 plant condition colours

Label the first module PLANT. The most severe air or soil condition wins: critical overrides warning, and warning overrides normal. Yellow uses red and green together.

| Colour | Meaning | User action |
| --- | --- | --- |
| Green · steady | Normal air and soil-moisture conditions, with all required sensors reporting valid data | Continue monitoring. |
| Yellow · steady | Warning air or soil-moisture condition, or soil calibration is required | Read the bottom OLED message and check the relevant condition. |
| Red · steady | Critical air/dry-soil condition, DHT22/BH1750 error, or DS18B20 soil-temperature error | Read the OLED message. Check the plant condition or the named sensor connection before acting. |
| Blue | Not used for plant status | Reserved for other uses; no current plant meaning. |
| Off | Possible briefly during startup | If it stays off, check power, wiring and the sketch. |

# 4 Current plant thresholds

| Reading | Green normal | Yellow warning | Red critical |
| --- | --- | --- | --- |
| Soil moisture | 40% to 100% | 20% to below 40% | Below 20% |
| Air temperature | 18°C to 30°C | 10°C to below 18°C; above 30°C to 35°C | Below 10°C or above 35°C |
| Air humidity | 40% to 70% RH | 25% to below 40%; above 70% to 85% | Below 25% or above 85% |

These are demonstration thresholds, not universal plant-care targets. DHT22 measures air temperature and humidity. BH1750 lux is displayed, but no low-light or high-light thresholds currently affect the plant LED. The DS18B20 measures soil temperature, but the current sketch does not apply plant-specific soil-temperature thresholds; a missing, invalid or stale DS18B20 reading is treated as a sensor fault and sets the plant status to red.

# 5 Soil percentage and OLED messages

The example calibration uses dry raw = 4095 and watered-and-drained raw = 1500. Replace them with measurements from your own probe and soil. Until then, the displayed “% demo” is an estimate. This is a relative moisture scale, not a laboratory water-content measurement.

| OLED message | What it means |
| --- | --- |
| NORMAL | All evaluated plant conditions are normal. |
| SOIL DRY / SOIL VERY DRY | Soil warning / soil critical. |
| AIR WARNING / AIR CRITICAL | Air temperature or humidity is outside the normal range. |
| AIR + SOIL WARNING / CRITICAL | Both air and soil have the named severity. |
| SOIL: CALIBRATE | Calibration is missing or invalid; normally yellow unless air is critical. |
| DHT22 ERROR / BH1750 ERROR / SENSOR ERROR | One or both environmental sensor readings have failed; red. |
| SOIL TEMP ERROR | DS18B20 reading is missing, invalid or stale; plant LED is red. |

Overwatering note: the current code has no too-wet threshold, so a 100% moisture reading can still show green. If the raw value stays at 4095, check the wiring and confirm the reading changes between dry and wet soil.

# 6 RGB 2 connection status colours

Label the second module CONNECTION. Its colours describe communications and are independent of the plant LED. In the current sketch MQTT is enabled, so after Wi-Fi connects the LED may be blue briefly while network time is synchronised, then green when AWS MQTT connects or yellow if an MQTT attempt fails.

| Colour and pattern | Meaning |
| --- | --- |
| Blue · blinking | Initial Wi-Fi connection attempt, during the first 30 seconds before the first successful connection. Approximately 0.5 seconds on and 0.5 seconds off. |
| Blue · steady | Wi-Fi connected before the first MQTT connection attempt, including the network-time synchronisation period. It would also be used if MQTT configuration were disabled or incomplete. |
| Green · steady | Wi-Fi and the MQTT broker are connected. |
| Yellow · steady | Wi-Fi connected, but MQTT is attempting to connect, retrying, failed or disconnected after an attempt. |
| Red · steady | Wi-Fi is disconnected after the initial 30-second window, or the connection has been lost. It can remain red while reconnecting. |
| Off | Possible briefly at boot. If persistent, check power, wiring and software. |

The current sketch has ENABLE_MQTT set to true and uses TLS on port 8883. With valid certificates and a successful AWS IoT connection, steady green is expected. Blue is mainly the pre-attempt/time-sync state; yellow means MQTT has been attempted but is not connected. Green confirms broker connectivity only; sensor publishing should still be checked in Serial output or the AWS/backend side.

# 7 Connection labels on the OLED

| OLED label | Meaning |
| --- | --- |
| W:OFF | Wi-Fi disconnected. |
| W:ON | Wi-Fi connected; MQTT not configured or disabled. |
| M:ON | MQTT connected. |
| M:OFF | MQTT configured but not connected. |

# 8 Final wiring and startup checks

Power the ESP32 through its USB connector. Use its 3V3 rail for the listed sensor modules, and connect every GND to a common ground. Do not feed a 5 V analogue signal into GPIO34.

Keep the soil interface board and electrical connectors dry. Only the probe prongs go into soil. Follow the printed board labels for the connector order.

Confirm that GPIO18, GPIO19 and GPIO23 are used only by RGB 2, and GPIO32 is reserved for the DS18B20 soil-temperature data line. Do not reuse these pins for a future pump or float switch.

Check breadboard rail continuity and tighten loose jumper wires. Reconnect USB and allow the sensors to start; readings normally update every 2.5 seconds.

Check that soil raw readings change before trusting the percentage. Calibrate using dry soil and watered soil after excess water has drained.

Check the PLANT colour against the OLED condition message, then check the CONNECTION colour against its Wi-Fi or MQTT label.

Confirm the DS18B20 changes from SOIL TEMP ERROR to a real temperature value and that the OLED shows “Soil T: … C”.

For cloud testing, wait for M:ON / a steady green CONNECTION LED, then confirm a JSON sensor payload is published about every 2.5 seconds.

# 9 Current AWS IoT / MQTT behavior

The current sketch is configured to use AWS IoT Core over secure MQTT/TLS. Credentials and private-key material are intentionally not reproduced in this guide.

| Setting / topic | Current code behavior |
| --- | --- |
| MQTT client ID | greenpulse_01 |
| Broker transport | TLS MQTT on port 8883; device certificate, private key and Amazon root CA are loaded into WiFiClientSecure. |
| Publish: greenpulse/sensors | Sends the sensor JSON payload when MQTT is connected. |
| Publish: greenpulse/status | Publishes “Device Online” after a successful broker connection. |
| Subscribe: greenpulse/ai/care | Incoming plant-care messages are printed to Serial. |
| Subscribe: greenpulse/ai/weather | Incoming weather/AI messages are printed to Serial. |
| Reconnect behavior | Wi-Fi retries roughly every 15 seconds; MQTT retries roughly every 15 seconds after time synchronisation is ready. |

Sensor JSON fields: device_id, sequence_number, soil_moisture (when calibration is valid), temperature and humidity (when DHT22 is valid), light_intensity (when BH1750 is valid), soil_temperature, and soil_temperature_ok. Invalid soil temperature is sent as null rather than a false numeric value.

Current display/update cycle: sensor readings, OLED status and MQTT payload generation run approximately every 2.5 seconds. The OLED shows GreenPulse, network state, air temperature, humidity, light, soil moisture/raw value, soil temperature and the final status message.

Security note: Do not place the Wi-Fi password or AWS private key in screenshots, reports or shared repositories. If a private key has been exposed outside the intended device, replace/rotate the corresponding IoT certificate and key.

Scope: this guide now covers the SSD1306 OLED, DHT22, BH1750, resistive soil-moisture sensor, DS18B20 soil-temperature probe, two HW-479 RGB modules, Wi-Fi status and the current AWS IoT MQTT behavior. Pump control and a water-level switch are not included in this sketch.